ZombieVerter VCU — Build How-To

Rob's Stm32-vcu fork with VESC inverter class  |  Last updated May 2026

# 1. Prerequisites

The following tools must be installed and working before you can build:

- ARM GNU Toolchain 13.x  —  extracted to C:\arm-none-eabi-13\

- Git for Windows  —  provides Git Bash shell

- GnuWin32 make or ezwinports make  —  installed via winget

- Python 3.x  —  installed, Windows Store alias disabled

- VS Code  —  with the repo open as a folder

Always use Git Bash (not PowerShell or CMD) to run make commands. PowerShell does not have the Unix utilities the Makefile requires.

# 2. First-Time Setup

Do this once after cloning the repo for the first time.

## 2a. Clone the repo

cd ~/Documents/GitHub

git clone --recursive https://github.com/damienmaguire/Stm32-vcu

cd Stm32-vcu

## 2b. Copy your modified files into the repo

Copy these 5 files from your previous working folder into the new clone:

- include/vescinv.h

- src/vescinv.cpp

- src/stm32_vcu.cpp

- include/param_prj.h

- Makefile

## 2c. Set up Git Bash PATH

Paste these lines into Git Bash to make the toolchain available. They are saved permanently in ~/.bashrc so you only need to do this once:

echo 'export PATH="/c/arm-none-eabi-13/bin:$PATH"' >> ~/.bashrc

echo 'export PATH="/c/Users/rober/AppData/Local/Microsoft/WinGet/Links:$PATH"' >> ~/.bashrc

echo 'export PATH="$HOME/bin:$PATH"' >> ~/.bashrc

source ~/.bashrc

## 2d. Create the python3 alias

libopencm3 requires python3 specifically. Create a symlink in ~/bin:

mkdir -p ~/bin

ln -s /c/Users/rober/AppData/Local/Programs/Python/Python312/python.exe ~/bin/python3

## 2e. Build libopencm3 (first time only)

make get-deps

This compiles the libopencm3 STM32F1 library. Takes 1-2 minutes. Only needs to be done once per clone.

# 3. Normal Build Workflow

Every time you make changes to source files, rebuild with:

make

That's it. Make only recompiles files that changed so incremental builds are fast.

## Other useful make commands

- make                   — build only changed files (normal use)

- make clean             — delete all compiled objects

- make clean && make     — full rebuild from scratch

# 4. Output Files

After a successful build you will see:

OBJCOPY stm32_vcu.bin

OBJCOPY stm32_vcu.hex

Both files are in the repo root folder. For flashing via the ZombieVerter ESP web interface, use:

stm32_vcu.bin

Connect to the ZombieVerter WiFi, open the web interface, navigate to firmware update, and upload stm32_vcu.bin.

# 5. VESC Inverter Configuration

After flashing, configure ZombieVerter and VESC Tool before first run:

## In VESC Tool (via USB, before connecting CAN)

- Run FOC motor detection wizard to completion

- App Settings → General → VESC ID: 1

- App Settings → General → App to use: No App

- App Settings → General → CAN Mode: VESC

- App Settings → General → CAN Status Rate 1: 20 Hz

- App Settings → General → CAN Status Mode 1: Status 1, 4, and 5 checked

- Write config to VESC, then disconnect USB and connect CAN bus

## In ZombieVerter Web UI

- Inverter type: 9 = VESC

- InverterCan: 0 = CAN1  or  1 = CAN2  (whichever the VESC is wired to)

- Save parameters and reboot

CAN bus wiring: 500 kbps, 120 ohm termination resistor at each end. ZombieVerter CAN H/L to VESC CAN H/L.

## Key tuning constants in include/vescinv.h

- VESC_CONTROLLER_ID    — must match VESC Tool VESC ID (default 1)

- VESC_MAX_CURRENT_A    — peak motor current in amps. Start at 5-10A for RC motors

- VESC_MAX_REGEN_A      — regen braking current. Start at 3-5A

- VESC_POLE_PAIRS       — read from VESC Tool after FOC detection

# 6. Troubleshooting

- 'make not found' in PowerShell  →  switch to Git Bash terminal

- 'arm-none-eabi-gcc not found'   →  run: export PATH="/c/arm-none-eabi-13/bin:$PATH"

- 'python3 not found'             →  re-run the ~/bin/python3 symlink step

- 'stdint.h not found'            →  wrong toolchain version. Must use 13.x not 15.x

- VESC not responding             →  check CAN termination, baud rate (500k), and VESC ID matches

- Motor spins wrong direction     →  set reversemotor=1 in ZombieVerter web UI